1. Resolution of computed radiography

2 lp/mm

2. Required monitor resolution to display full resolution chest radiograph

5 MP

3. Resolution of computed mammography

10 lp/mm

4. Detector for nuclear thyroid studies

NaI

5. Tc99 MAA maximum dose is to what organ?

Lung

6. Tc99 MDP/pyrophosphate maximum dose is to what organ?

Bone marrow

7. Doppler shift of 5 MHz at 60 degree angle?

100- 1000 Hz

8. Mammo grid ratio?

5:1

9. Longest to shortest half life

Xenon, aerosolized, MAA

10. Speed of sound in lung and soft tissues?

330 and 1540 m/s

11. Detector for direct fluoroscopy?

Selenium

12. Which is not a sequence for gradient recall echo imaging?

Inversion recovery/FLAIR

GRASS

FLASH

FISP

13. Blood flow can cause which of the following?

Increased T1 relaxation

Decreased T1 relaxation

Enhanced signal

14. Larmor frequency

63 MHz at 1.5 Tessla

15. Required wall shielding

1/16 of an inch

16. Ring artifacts in MRI are due to?

Undersampling

17. CT dose efficiency is best expressed as?

MSAD

CTDI

CTDI 100

CTDI FDA

18. Regarding the # of protons and neutrons in a nucleus, if N/Z=1, at what proton # does the ratio not equal 1?

15

30

90

120

19. What percent of xrays are characteristic

5-10 %

20. Best geometric detection efficiency

Anger camera

Digital camera

Uptake probe

Well counter

21. Low contrast images are most affected by?

Noise

22. What does MIRD stand for?

Medical internal radiation dose

23. Multi head SPECT does what?

faster imaging

24. Stem on anode is made from what?

copper

25. Max safe MR power

4W/kg

26. Quadruple the distance (1m to 4m) decreases dose to what percent of original?

50%

25%

12.5%

6.25%

27. Energy of electron in keV

33

28. Average background radiation

3 mSv/ year or 300 mRem/ year

29. Dose limit for general public

1 mSv/ year or 100 mRem/ year

30. Dose limit for radiation worker

50 mSv/ year or 500 mRem/ year

- Wavelength of conventional x-ray

0.1- 5 nm or 10- 50 angstroms

- Maximum dose for nuclear medicine study

100 mGy/ min

- Nyquist frequency

1/2

- Leakage radiation

1 mGy/hour at 1 Meter

- Specific gamma ray constant

Rad x cm^2/ mCi x hr

- Most common interaction of diagnostic xrays

compton

- Average glandular dose for mammo

0.5 cGy

- Order of increasing receptor exposure

Fluoro, cine, photospot, screen film, mammo

- Fetus may have congenital abnormalities if exposed to radiation at?

20-40 days

- Skin erythema occurs at what dose?

2 Gy or 200 Rads

- Benefit of multidetector CT

Decreased voxel size

- If you double MRI slice thickness, what will it do to the SNR?

Double

- What does the FDA no longer require?

Aperture

- What is the definition of CT pitch?

Movement / Thickness

- MRI slice location is determined by?

Gradient strength

Receiver coil strength

Frequency encoding gradient

Radiofrequency

- Barrel artifact is caused by?

Non-linear gradient

47. All of the following increase SNR except

Decreasing slice thickness

48. Intrinsic resolution of a gamma camera is tested with

Tc99 point source x 5 FOVU

- Half value layer for soft tissues

30 mm

- Which of the following will increase SNR in MRI?

Stronger primary magnetic field

- Quantum detector efficiency definition

Percent of photons that interact with the screen

- Effective half life definition

1/effective half life= 1/biologic half life + 1/nuclear half life

- Receptor exposure for fluoroscopic vs fluorographic image

- Fluoro resolution

1024 x 1024

- Effective dose for CT head

1-2 mSv

- Power loading

- A 4 view mammogram is equal to

1 year of background radiation

- Maximum dose for single view mammogram

3 mSv

- FWHM of SPECT vs PET

10mm vs 5mm

- A thicker screen will have what effect on quantum mottle

none

- Inversion recovery sequence on MRI will do what

Enhance T1 differences

- Chemical shift artifact is caused by

Fat shifting in the frequency encoding gradient

- Aliasing artifact can be corrected by

Increasing the sampling rate

- Aliasing artifact occurs when

Signal wraps around from high to low

- Lowest to highest receptor exposure

Photospot, Screen film, Mammo

- PET detector BGO

- Enhance T2 in spin echo by

- Which is true for computed radiography when compared to screen film

Fewer dark images

Fewer artifacts

Fewer redoes

Less noise

- What difference in image contrast can CT detect

0.5% or 5 HU

- In PET imaging, if the distance to event is increased

Spatial resolution is decreased

- Skin exposure for abdominal radiograph

300 mRad

- Skin exposure for chest radiograph

20 mRad

- Inherent resolution of nuclear medicine

4 mm

- What is used to detect shallow doses of radiation

TLD ring

- Converging collimators do what to resolution and efficiency

- Risk of cancer from radiation

4% per Gy

- Low contrast lesion

- Flat panel detectors

Increase spatial resolution

Decrease noise

Decrease dose

- How many CT images can be displayed on a 5 MP monitor

20

- Maximum organ dose for Nuc med study

50 mGy

- Detector for indirect fluoroscopy

CSI

- Partial volume effect

Averaging linear attenuation coefficient in a voxel

- Attenuation of 5 MHz ultrasound in soft tissue

2 dB

- What is the pixel size for computed mammography?

0.05 mm

- What is the maximum dose for high-dose fluoroscopy?

20 Rads/ minute

- Increasing kVp in mammography will do what to contrast and patient dose?

Decrease contrast, decrease dose.

- Rhodium filters out photons above what energy?

25 keV

- What is the maximum allowable energy of Ultrasound in mammals

100 mW/ cm^2

- Information contained in the center of k space has

lower spatial frequency

- What is the most important factor concerning grids?

Grid ratio

- PET sensitivity increases with

Detector efficiency